Khang Do

a)

Passed frequency: 17 kHz.

Stopped frequencies: 10 kHz and 25 kHz

b) Stage 1 (b = 0.2)

Q = 3.8

Op-amp gain per stage (non-inverting):
k = b A Q (b < 1)
A = 1 + Rf/Rg

If you keep the same op-amp gain as before:
Choose Rg = 10 kΩ, Rf = 17.4 kΩ
A = 1 + 17.4k/10k = 2.74

Stage 1 gain at resonance (linear):
k1 = b A Q = 0.2(2.74)(3.8) = 2.08 (about 6.36 dB)

BW = f0 / Q = 17000 / 3.8 = 4473.68 Hz

The −3 dB cutoff frequencies for a standard 2nd-order band-pass are:
fL = f0 ( sqrt(1 + 1/(4Q^2)) − 1/(2Q) ) ≈ 14.91 kHz
fH = f0 ( sqrt(1 + 1/(4Q^2)) + 1/(2Q) ) ≈ 19.38 kHz

C = 15 nF
R = 624 Ω

Stage 1 b-resistors:
R/b = 624/0.2 = 3120 Ω
R/(1−b) = 624/0.8 = 780 Ω
2R = 1248 Ω
R = 624 Ω

Stopband gain (relative to peak) for ONE stage:
At 10 kHz: 0.2299 → about −12.77 dB relative to peak
At 25 kHz: 0.3159 → about −10.01 dB relative to peak

Stage 2 (b = 0.2)

Same as Stage 1 (if identical):
Q = 3.8
Rg1 = 10 kΩ, Rf1 = 17.4 kΩ ⇒ A2 = 2.74
k2 = 0.2(2.74)(3.8) = 2.08 (about 6.36 dB)

BW = 4473.68 Hz
fL ≈ 14.91 kHz
fH ≈ 19.38 kHz

C = 15 nF
R = 624 Ω

Stage 2 b-resistors:
R/b = 3120 Ω
R/(1−b) = 780 Ω
2R = 1248 Ω
R = 624 Ω

Stopband gain (relative to peak) for ONE stage:
At 10 kHz: 0.2299 → about −12.77 dB relative to peak
At 25 kHz: 0.3159 → about −10.01 dB relative to peak

c)

Htotal(s) = (3.42775×10^9 s^2) /
(s^4 + 5.62303×10^4 s^3 + 2.36190×10^10 s^2 + 6.41828×10^14 s + 1.30286×10^20)clc; clear; close all;

s = tf('s');

R = 624;

C = 15e-9;

b  = 0.2;

Rf = 17.4e3;

Rg = 10e3;

A  = 1 + Rf/Rg;

f0 = 1/(2*pi*R*C);

w0 = 2*pi*f0;

Q  = 3.8;

k  = b*A*Q;

Hstage = k * ((w0/Q)*s) / (s^2 + (w0/Q)*s + w0^2);

Htot = Hstage^2;

opts = bodeoptions;

opts.FreqUnits = 'Hz';

opts.XLim = [1e3 1e5];

figure;

bodeplot(Htot, opts);

grid on;

f = logspace(log10(1e3), log10(1e5), 4000);

w = 2*pi*f;

[mag,~] = bode(Htot, w);

mag = squeeze(mag);

mag_dB = 20*log10(mag);

[pk_dB, idx] = max(mag_dB);

f_pk = f(idx);

fprintf('Peak: %.2f Hz, %.2f dB\n', f_pk, pk_dB);

f_test = [10e3 17e3 25e3];

w_test = 2*pi*f_test;

[magT,~] = bode(Htot, w_test);

magT = squeeze(magT);

gain_dB = 20*log10(magT);

fprintf('At 10 kHz: %.2f dB\n', gain_dB(1));

fprintf('At 17 kHz: %.2f dB\n', gain_dB(2));

fprintf('At 25 kHz: %.2f dB\n', gain_dB(3));

fprintf('17k - 10k difference: %.2f dB\n', gain_dB(2) - gain_dB(1));

fprintf('17k - 25k difference: %.2f dB\n', gain_dB(2) - gain_dB(3));

d)

e)

|  | MatLab | Physical | LTSpice |
| --- | --- | --- | --- |
| 10kHz | -12.7 |  | -12.64 |
| 17kHz | 12.74 |  | 12.9 |
| 25kHz | -7.27 |  | -7.62 |

f)